Network Automation Bootcamp

Lab Guide — Git · Ansible · REST API

Version 2.0   |   May 2026   |   10–12 Students

# 1. Introduction

The Network Automation Bootcamp is a hands-on lab designed for 10–12 network engineers. It covers three core pillars of modern network automation: Git, Ansible, and REST APIs. Students work on shared lab devices (two routers and one switch), a local NetBox instance as the source of truth, and several free public APIs to explore real-world data — no accounts or credit cards required.

## 1.1 Learning Objectives

Track and roll back network configs using Git branches and merge requests.

Write Ansible playbooks to configure routers and a switch at scale with Jinja2 templates.

Consume the NetBox REST API to query and update the source of truth.

Query free public APIs (BGPView, PeeringDB, Cisco DevNet sandbox) to explore real BGP data.

Connect all three tools into a repeatable, auditable end-to-end change pipeline.

## 1.2 Student Setup

Each student receives:

A workstation IP in 10.0.0.101–112 (Pod-01 to Pod-12)

A personal Git branch:  student/pod-XX-yourname

A personal NetBox API token for the lab duration

SSH credentials for the shared lab devices

git clone http://10.0.0.60/bootcamp/Network-Automation-Bootcamp.git
cd Network-Automation-Bootcamp
git checkout -b student/pod-XX-yourname
pip install ansible requests netmiko napalm pynetbox

## 1.3 Prerequisites

| Requirement | Version | Purpose |
| --- | --- | --- |
| Git | ≥ 2.30 | Source control for network configs |
| Python | ≥ 3.9 | Scripting and API interaction |
| Ansible | ≥ 2.14 | Configuration management engine |
| requests | latest | HTTP client for REST/RESTCONF |
| netmiko | latest | Multi-vendor SSH sessions |
| napalm | latest | Device abstraction layer |
| pynetbox | latest | NetBox API Python wrapper |

# 2. Lab Architecture & Topology

The lab is divided into three networks: the classroom management network (10.0.0.0/24) where students, the control node, NetBox, and the Git server live; the device network (10.0.1.0/24) for the routers and switch; and the internet for free public API access.

INTERNET
                             │
          ───────────────────┼──────────────────────
            FREE PUBLIC APIs (no account needed)
          ───────────────────┼──────────────────────
          │                  │                      │
  Cisco DevNet IOS-XE   BGPView API          PeeringDB
  (sandbox free)         (free)               (free)
          │                  │                      │
          └──────────────────┼──────────────────────┘
                             │ HTTPS
═════════════════════════════╪══════════════════════════
         MGMT NETWORK  10.0.0.0/24
═════════════════════════════╪══════════════════════════
                             │
        ┌────────────────────┼───────────────┐
        │                    │               │
 CONTROL NODE          NETBOX           GIT SERVER
 10.0.0.50             10.0.0.100       10.0.0.60
 Git/Ansible/Python    :8000 (IPAM)     (Gitea)
        │                    │
        └──────────┬─────────┘
                   │ SSH / RESTCONF / Ansible
═══════════════════╪══════════════════════════════
         DEVICE NETWORK  10.0.1.0/24
═══════════════════╪══════════════════════════════
       ┌───────────┼──────────────┐
  ROUTER-1    ROUTER-2       SWITCH-1
  10.0.1.1    10.0.1.2       10.0.1.10
  IOS-XE      NX-OS          IOS  VLANs

═══════════════════════════════════════════════════
  STUDENT PODS  10.0.0.101 – 10.0.0.112  (×12)
═══════════════════════════════════════════════════

## 2.1 IP Address Plan

| Device / Host | IP Address | Role |
| --- | --- | --- |
| Control Node | 10.0.0.50 | Automation engine (shared) |
| NetBox | 10.0.0.100 | IPAM / DCIM + REST API |
| Gitea (Git Srv) | 10.0.0.60 | Repository server |
| Router-1 | 10.0.1.1 | IOS-XE lab router + RESTCONF |
| Router-2 | 10.0.1.2 | NX-OS secondary router |
| Switch-1 | 10.0.1.10 | IOS access switch + VLANs |
| Student Pod 01–12 | 10.0.0.101–112 | Student workstations |

## 2.2 Free Public API Reference

| Service | Base URL | Auth | Best For |
| --- | --- | --- | --- |
| Cisco DevNet IOS-XE | sandbox-iosxe-latest-1.cisco.com | devnetuser/Cisco123! | RESTCONF on real IOS-XE |
| NetBox Demo | demo.netbox.dev/api/ | Public demo token | IPAM API queries |
| BGPView | api.bgpview.io | None | ASN, prefix, BGP data |
| PeeringDB | peeringdb.com/api/ | None (read) | IX and peering records |
| ipinfo.io | ipinfo.io | None (50 k/mo) | IP geolocation & ASN |
| httpbin.org | httpbin.org | None | HTTP method practice |

# 3. Section 1 — Git for Network Automation

Git is the single source of truth for all device configurations. Every change is tracked, attributed, and reversible. Students work on personal branches and submit merge requests to main, mirroring real change-management workflows.

## 3.1 Exercise 1 — Initialise a Config Repository

mkdir net-configs && cd net-configs
git init
git config user.name  'Network Engineer'
git config user.email 'neteng@example.com'
cp ../configs/router1_baseline.cfg .
git add router1_baseline.cfg
git commit -m 'feat: add Router-1 baseline config'
git log --oneline

## 3.2 Exercise 2 — Branching for Change Management

git checkout -b change/CR-1042-add-loopback
echo 'interface Loopback0\n ip address 10.255.255.1 255.255.255.255' >> router1.cfg
git add router1.cfg
git commit -m 'feat(CR-1042): add Loopback0 to Router-1'
git diff main change/CR-1042-add-loopback
git checkout main
git merge --no-ff change/CR-1042-add-loopback

## 3.3 Exercise 3 — Configuration Rollback

echo 'no ip routing' >> router1.cfg
git add router1.cfg && git commit -m 'ERROR: removed ip routing'
git log --oneline
git revert HEAD          # safe: creates an undo commit
git push origin main

## 3.4 Key Git Commands

| Command | Purpose |
| --- | --- |
| git init | Initialise a new repository |
| git clone <url> | Clone a remote repository |
| git add <file> | Stage changes |
| git commit -m 'msg' | Commit staged changes |
| git log --oneline | Compact commit history |
| git checkout -b <branch> | Create and switch to new branch |
| git merge --no-ff <branch> | Merge with a merge commit |
| git revert HEAD | Undo last commit (safe) |
| git tag v1.0 | Tag a golden config release |

# 4. Section 2 — Ansible for Network Automation

Ansible connects to the lab routers and switch over SSH and applies declarative YAML playbooks. Students always run with --check --diff before applying, mirroring the peer-review gate used in production.

## 4.1 Lab Devices

| Host | IP | OS | Role |
| --- | --- | --- | --- |
| router1 | 10.0.1.1 | IOS-XE | Core router / OSPF / RESTCONF |
| router2 | 10.0.1.2 | NX-OS | Secondary router |
| switch1 | 10.0.1.10 | IOS | Access switch / VLANs |

## 4.2 Inventory File (hosts.ini)

[routers]
router1 ansible_host=10.0.1.1
router2 ansible_host=10.0.1.2

[switches]
switch1 ansible_host=10.0.1.10

[network:children]
routers
switches

[network:vars]
ansible_user=admin
ansible_password=Lab@1234
ansible_network_os=ios
ansible_connection=network_cli

## 4.3 Playbook 1 — Gather Facts

ansible-playbook -i inventory/hosts.ini playbooks/01-gather-facts.yml

## 4.4 Playbook 2 — Push Interface Config

ansible-playbook -i inventory/hosts.ini playbooks/02-push-config.yml --check --diff
ansible-playbook -i inventory/hosts.ini playbooks/02-push-config.yml

## 4.5 Playbook 3 — Compliance Check

ansible-playbook -i inventory/hosts.ini playbooks/03-compliance-check.yml

## 4.6 Jinja2 Template Snippet

{% for iface in interfaces %}
interface {{ iface.name }}
 description {{ iface.description }}
 {% if iface.ip is defined %}
 ip address {{ iface.ip }} {{ iface.mask }}
 {% endif %}
 {% if iface.shutdown %}shutdown{% else %}no shutdown{% endif %}
!
{% endfor %}

# 5. Section 3 — REST API for Network Automation

This section uses two types of APIs: the local NetBox REST API as a source of truth, and several free, no-account public APIs for exploring real-world routing and network data.

## 5.1 Script 1 — NetBox: Query Devices

Fetches active devices from the local NetBox instance and prints a summary table. Students then extend the script to POST a new device via the API.

NETBOX_URL   = 'http://10.0.0.100:8000'
NETBOX_TOKEN = 'your_token_here'

headers = {'Authorization': f'Token {NETBOX_TOKEN}'}
resp = requests.get(f'{NETBOX_URL}/api/dcim/devices/?status=active', headers=headers)
devices = resp.json()['results']

## 5.2 Script 2 — RESTCONF: Push Interface Config

Uses RESTCONF (RFC 8040) and HTTP PUT to configure GigabitEthernet0/1 on Router-1 with a description and IP address using the ietf-interfaces YANG model.

## 5.3 Script 3 — BGPView: Real Routing Data (Free, No Auth)

Queries the BGPView public API to look up ASN information and IPv4 prefixes. Students use Cloudflare (AS13335) as a starting point, then look up their own ISP.

# Look up Cloudflare (no auth required)
resp = requests.get('https://api.bgpview.io/asn/13335')
data = resp.json()['data']
print(data['name'], data['country_code'])

# List their prefixes
resp = requests.get('https://api.bgpview.io/asn/13335/prefixes')
prefixes = resp.json()['data']['ipv4_prefixes']

## 5.4 Script 4 — Cisco DevNet Always-On Sandbox (Free)

Connects to Cisco's free, always-on IOS-XE sandbox using RESTCONF. No sign-up required. Students query interfaces, then compare the YANG model output to show ip interface brief.

SANDBOX = 'sandbox-iosxe-latest-1.cisco.com'
auth    = HTTPBasicAuth('devnetuser', 'Cisco123!')
url     = f'https://{SANDBOX}/restconf/data/ietf-interfaces:interfaces'
resp    = requests.get(url, auth=auth, headers=HEADERS, verify=False)
interfaces = resp.json()['ietf-interfaces:interfaces']['interface']

## 5.5 Other Free APIs to Explore

| API | Example Call | Returns |
| --- | --- | --- |
| PeeringDB | GET /api/ix?name=AMS-IX | Internet exchange details |
| ipinfo.io | GET https://ipinfo.io/8.8.8.8 | IP geo, ASN, org |
| httpbin.org | POST /post (any JSON body) | Echo of your request |

# 6. End-to-End Automation Pipeline

Combining all three sections into a repeatable, auditable change workflow:

CHANGE REQUEST
      │
  git checkout -b change/CR-XXXX      ← branch from main
      │
  Edit YAML vars / Jinja2 template    ← define desired state
      │
  ansible-playbook --check --diff     ← preview diff
      │
  Pull Request → Peer Review          ← human gate
      │
  git merge --no-ff                   ← merge to main
      │
  ansible-playbook (apply)            ← push to devices
      │
  python3 01_netbox_get_devices.py    ← verify via NetBox API
      │
  git tag v1.x                        ← tag golden state

# 7. Lab Completion Checklist

| Section | Criteria | Done? |
| --- | --- | --- |
| Git | Repo initialised with at least one config file | ☐ |
| Git | At least two commits in git log | ☐ |
| Git | Feature branch created and merged via merge request | ☐ |
| Git | Config rollback demonstrated with git revert | ☐ |
| Ansible | Inventory with routers + switches groups | ☐ |
| Ansible | gather-facts playbook runs on all devices | ☐ |
| Ansible | push-config applies change via Jinja2 template | ☐ |
| Ansible | compliance-check generates a report | ☐ |
| Ansible | --check --diff used before every real push | ☐ |
| API | Script 1 queries NetBox and adds a device via POST | ☐ |
| API | Script 2 applies interface config via RESTCONF PUT | ☐ |
| API | Script 3 looks up an ASN and parses BGP prefixes | ☐ |
| API | Script 4 queries Cisco DevNet sandbox interfaces | ☐ |
| Pipeline | Full CR-to-deploy pipeline demonstrated end-to-end | ☐ |

Network Automation Bootcamp v2.0  ·  May 2026  ·  10–12 Students